CALMORB System Application GitHub:

Mobile Application:

ImAzimm/Calm-Orb-Mobile-Apps

Hardware Configuration:

Nipz652/CalmOrbIoT: Configuration of CalmOrb's IoT system

============================================================================

STRESS BALL MONITORING SYSTEM - QUICK START GUIDE

============================================================================

This guide will help you set up and start the autism child stress monitoring system from scratch.

============================================================================

PART 1: HARDWARE REQUIREMENTS

============================================================================

Required Components:

--------------------

[1] Raspberry Pi 4 or 5 (4GB RAM recommended)

[2] MicroSD card (16GB minimum, Class 10)

[3] ESP32 Stress Ball (pre-configured with firmware) (DfPlayer Mini, FSR402, MPU6050, Speaker)

[4] ILI9341 2.8" TFT Display (connected to Pi SPI)

[5] INMP441 I2S Microphone (connected to Pi GPIO 18,19,20)

[6] DHT22 Temperature Sensor (connected to Pi GPIO 4)

[7] USB Webcam (any compatible model)

[8] M5Stack Atom Echo (optional, for voice feedback)

[9] Power supply for Raspberry Pi (5V 3A USB-C)

[10] Mobile device (Android/iOS) with BLE support

[11] Wifi Dongle (For better connectivity with esp32 at wlan0)

============================================================================

PART 2: SOFTWARE REQUIREMENTS

============================================================================

Operating System:

-----------------

- Raspberry Pi OS (64-bit recommended)

- Tested on: Raspberry Pi OS Bookworm (Debian 12)

Python Version:

---------------

- Python 3.9 or higher (3.13 tested and working)

Internet Connection:

--------------------

- Required for initial setup and package installation

- WiFi recommended (will create AP for ESP32 communication)

============================================================================

PART 3: INITIAL RASPBERRY PI SETUP

============================================================================

Step 1: Flash Raspberry Pi OS

------------------------------

1. Download Raspberry Pi Imager: https://www.raspberrypi.com/software/

2. Insert microSD card into your computer

3. Open Raspberry Pi Imager

4. Choose OS: Raspberry Pi OS (64-bit)

5. Choose Storage: Your microSD card

6. Click Settings (gear icon):

- Set hostname: yourhostname

- Enable SSH (use password authentication)

- Set username: your username

- Set password: [your password]

- Configure WiFi (optional)

7. Click "Write" and wait for completion

Step 2: Boot Raspberry Pi

--------------------------

1. Insert microSD card into Raspberry Pi

2. Connect power supply

3. Wait 2-3 minutes for first boot

4. Find IP address:

- Option 1: Check your router's DHCP clients

- Option 2: Use: ping yourhostname

- Option 3: Connect monitor and run: hostname -I

Step 3: SSH into Raspberry Pi

------------------------------

From your computer terminal:

ssh your username@ yourhostname

# OR

ssh your username@[IP_ADDRESS]

Enter your password when prompted.

============================================================================

PART 4: SYSTEM INSTALLATION

============================================================================

Step 1: Update System Packages

-------------------------------

Run these commands on the Raspberry Pi:

sudo apt update

sudo apt upgrade -y

Step 2: Install System Dependencies

------------------------------------

sudo apt install -y python3-pip python3-venv git

sudo apt install -y libasound2-dev i2c-tools

sudo apt install -y libatlas-base-dev

Step 3: Clone Project (if from Git)

------------------------------------

If you have the project on GitHub:

cd your directory

git clone [YOUR_REPO_URL] fyp2

cd fyp2

If you have the files locally, copy them to your directory

Step 4: Create Python Virtual Environment

------------------------------------------

cd "Your directory"

python3 -m venv ~/tftenv

source ~/tftenv/bin/activate

Your prompt should now show "(tftenv)"

Step 5: Install Python Dependencies

------------------------------------

pip install --upgrade pip

pip install -r requirements.txt

This will install:

- bleak (BLE communication)

- Pillow (image processing)

- opencv-python (camera)

- requests (Roboflow API)

- pyalsaaudio (audio monitoring)

- numpy (data processing)

- and other required packages

Step 6: Enable Hardware Interfaces

-----------------------------------

Enable I2C (for sensors):

sudo raspi-config nonint do_i2c 0

Enable I2S (for INMP441 microphone):

sudo nano /boot/firmware/config.txt

Add these lines at the end:

dtoverlay=i2s-mmap

dtoverlay=googlevoicehat-soundcard

Save (Ctrl+O, Enter) and exit (Ctrl+X)

Step 7: Reboot Raspberry Pi

----------------------------

sudo reboot

Wait 1-2 minutes, then reconnect via SSH.

============================================================================

PART 5: VERIFY HARDWARE CONNECTIONS

============================================================================

After reboot, verify all hardware is detected:

Check I2S Microphone:

---------------------

arecord -l

You should see:

card 1: sndrpigooglevoi [snd_rpi_googlevoicehat_soundcar], device 0

Check USB Webcam:

-----------------

ls /dev/video*

You should see: /dev/video0

Check TFT Display:

------------------

ls /dev/spidev*

You should see: /dev/spidev0.0 and /dev/spidev0.1

============================================================================

PART 6: CONFIGURE THE SYSTEM

============================================================================

Step 1: Edit Configuration (Optional)

--------------------------------------

nano /setting/settings.py

Important settings:

ESP32_IP = "192.168.4.1"                    # ESP32 stress ball IP

UDP_LISTEN_PORT = 4210                      # Port for ESP32 data

NOISE_MONITOR_DEVICE = "hw:sndrpigooglevoi,0"  # I2S microphone

CAMERA_DEVICE_INDEX = 0                     # USB webcam

DHT22_PIN = 4                               # Temperature sensor GPIO

Save (Ctrl+O, Enter) and exit (Ctrl+X)

Step 2: Create API Keys File

-----------------------------

nano setting/api_keys.py

Add your Roboflow API key:

ROBOFLOW_API_KEY = "your_api_key_here"

ROBOFLOW_MODEL_ID = "autism-ximav/1"

Save and exit.

============================================================================

PART 7: SETUP ESP32 STRESS BALL

============================================================================

The ESP32 stress ball should be pre-configured with firmware. To use it:

Step 1: Power On ESP32

-----------------------

1. Charge the ESP32 stress ball (USB-C cable)

2. Turn on the power switch

3. Wait for startup (LED should blink)

Step 2: Connect to ESP32 WiFi

------------------------------

The ESP32 creates a WiFi Access Point:

SSID: ESP32-StressBall

Password: 12345678

The Raspberry Pi will automatically connect when the system starts.

Step 3: Verify ESP32 Connection

--------------------------------

After connecting, ping the ESP32:

ping 192.168.4.1

You should see replies.

============================================================================

PART 8: SETUP SYSTEMD SERVICE (Auto-Start)

============================================================================

Step 1: Create Service File

----------------------------

sudo nano /etc/systemd/system/distress.service

Paste this content:

[Unit]

Description=Autism Stress Ball Monitoring System

After=network.target bluetooth.target

[Service]

Type=simple

User=yourname

WorkingDirectory=your directory

Environment="PATH=tftenv/bin:/usr/bin"

Environment="PYTHONUNBUFFERED=1"

ExecStart=tftenv/bin/python3 /your directory/services/main_service.py

Restart=on-failure

RestartSec=10

StandardOutput=journal

StandardError=journal

[Install]

WantedBy=multi-user.target

Save and exit.

Step 2: Enable and Start Service

---------------------------------

sudo systemctl daemon-reload

sudo systemctl enable distress.service

sudo systemctl start distress.service

Step 3: Check Service Status

-----------------------------

sudo systemctl status distress.service

You should see: "Active: active (running)"

============================================================================

PART 9: MONITOR THE SYSTEM

============================================================================

View Real-Time Logs:

--------------------

sudo journalctl -u distress.service -f

Press Ctrl+C to stop viewing logs.

View Last 100 Log Lines:

------------------------

sudo journalctl -u distress.service -n 100

View Logs Since Last Boot:

---------------------------

sudo journalctl -u distress.service -b

Filter Specific Service Logs:

------------------------------

sudo journalctl -u distress.service -f | grep "BLE"

sudo journalctl -u distress.service -f | grep "Noise"

sudo journalctl -u distress.service -f | grep "Camera"

Check System Status:

--------------------

# Check if Bluetooth is working

bluetoothctl

power on

discoverable on

exit

# Check if WiFi is connected to ESP32

iwconfig

# Check network connections

ip addr show

============================================================================

PART 10: CONTROL THE SERVICE

============================================================================

Start Service:

--------------

sudo systemctl start distress.service

Stop Service:

-------------

sudo systemctl stop distress.service

Restart Service:

----------------

sudo systemctl restart distress.service

Disable Auto-Start:

-------------------

sudo systemctl disable distress.service

Enable Auto-Start:

------------------

sudo systemctl enable distress.service

============================================================================

PART 11: CONNECT MOBILE APP

============================================================================

Step 1: Install Mobile App from GitHub

---------------------------

Install the companion mobile app (Android/iOS) on your phone.

Step 2: Enable Bluetooth

-------------------------

Make sure Bluetooth is enabled on your mobile device.

Step 3: Scan for Device

------------------------

1. Open the mobile app

2. Go to "Add Device" or "Scan"

3. Look for device named: "StressBallHub" or "CalmOrbHub" or What you have configured

4. Tap to connect

Step 4: Pair Child Profile

---------------------------

1. After connecting, go to Manage Child

2. Enable "Child Profile"

3. This will start all monitoring services:

- Camera AI behavior detection

- Environmental noise monitoring

- Temperature/humidity monitoring

- BLE proximity tracking

============================================================================

PART 12: TROUBLESHOOTING

============================================================================

Problem: Service won't start

-----------------------------

Solution:

# Check for errors

sudo journalctl -u distress.service -n 50

# Check Python path

which python3

/home/abdul/tftenv/bin/python3

# Activate virtual environment and test manually

source ~/tftenv/bin/activate

cd /home/abdul/fyp2

python3 services/main_service.py

Problem: No BLE connection

---------------------------

Solution:

# Restart Bluetooth

sudo systemctl restart bluetooth

# Check Bluetooth status

bluetoothctl

power on

discoverable on

pairable off

exit

# Restart service

sudo systemctl restart distress.service

Problem: ESP32 not sending data

--------------------------------

Solution:

# Check WiFi connection

iwconfig

# Should see: wlan0 connected to "ESP32-StressBall

# Check if ESP32 is reachable

ping 192.168.4.1

# Check UDP port

sudo journalctl -u distress.service -f | grep "UDP"

# Restart ESP32 (turn off/on power switch)

Problem: Camera not working

----------------------------

Solution:

# Check camera device

ls /dev/video*

# Test camera

sudo apt install fswebcam

fswebcam test.jpg

# Check logs

sudo journalctl -u distress.service -f | grep "Camera"

Problem: Noise monitor shows 0.0 dB

------------------------------------

Solution:

# Check I2S device

arecord -l

cat /proc/asound/cards

# Should see: googlevoicehat-soundcard

# Test recording

arecord -D hw:sndrpigooglevoi,0 -f S32_LE -r 16000 -c 2 -d 5 test.wav

# Check config

nano /home/abdul/fyp2/config/settings.py

# Verify: NOISE_MONITOR_DEVICE = "hw:sndrpigooglevoi,0"

# Restart service

sudo systemctl restart distress.service

Problem: Temperature sensor not working

----------------------------------------

Solution:

# Check GPIO

gpio readall

# Check DHT22 connection (GPIO 4)

# Check logs

sudo journalctl -u distress.service -f | grep "Temperature"

Problem: High CPU usage

-----------------------

Solution:

# Check CPU usage

top

# Press 'P' to sort by CPU

# If camera service is high, reduce FPS:

nano /home/abdul/fyp2/config/settings.py

# Change: CAMERA_DETECTION_INTERVAL = 0.5  # From 0.5 to 1.0

Problem: System crashes or freezes

-----------------------------------

Solution:

# Check memory usage

free -h

# Check logs before crash

sudo journalctl -u distress.service --since "1 hour ago"

# Increase swap space if low memory

sudo dphys-swapfile swapoff

sudo nano /etc/dphys-swapfile

# Change: CONF_SWAPSIZE=2048

sudo dphys-swapfile setup

sudo dphys-swapfile swapon

============================================================================

PART 13: DAILY OPERATIONS

============================================================================

Starting Your Day:

------------------

1. Turn on Raspberry Pi (if shut down)

2. Wait 2-3 minutes for boot

3. Turn on ESP32 stress ball

4. Service should auto-start and connect

5. Open mobile app and connect

6. Enable "Child Profile" to start monitoring

Shutting Down:

--------------

1. Disable "Child Profile" in mobile app (optional, saves battery)

2. Turn off ESP32 stress ball

3. Shutdown Raspberry Pi properly:

ssh abdul@raspberrypi.local

sudo shutdown -h now

Never just unplug the Raspberry Pi - always use shutdown command!

Viewing Data:

-------------

All sensor data is available in the mobile app:

- Real-time pressure and motion (from esp32)

- Environmental noise levels

- Temperature and humidity

- Proximity status (NEAR/MEDIUM/FAR/OUT_OF_RANGE)

- AI behavior detections

- Distress alerts

Checking Logs Remotely:

-----------------------

ssh abdul@raspberrypi.local

sudo journalctl -u distress.service -f

============================================================================

PART 14: UPDATING THE SYSTEM

============================================================================

Update System Packages:

------------------------

sudo apt update

sudo apt upgrade -y

Update Python Dependencies:

---------------------------

source ~/tftenv/bin/activate

pip install --upgrade pip

pip install -r requirements.txt --upgrade

Update Project Code (from Git):

--------------------------------

cd your directory

git pull origin main

sudo systemctl restart distress.service

============================================================================

PART 15: BACKUP AND RESTORE

============================================================================

Backup Configuration:

---------------------

cd your directory

tar -czf backup-$(date +%Y%m%d).tar.gz config/

Copy backup to your computer:

scp raspberrypi's ip@raspberrypi.local:your directory/backup-*.tar.gz .

Restore Configuration:

----------------------

cd your directory

tar -xzf backup-YYYYMMDD.tar.gz

============================================================================

PART 16: PERFORMANCE OPTIMIZATION

============================================================================

For Better Performance:

-----------------------

1. Use Raspberry Pi 4/5 (not 3)

2. Use high-quality SD card (Class 10 or better)

3. Ensure good cooling (heatsink or fan)

4. Keep system updated

5. Disable unnecessary services

To Reduce CPU Usage:

--------------------

Edit setting/settings.py:

CAMERA_DETECTION_INTERVAL = 1.0     # Increase from 0.5

SENSOR_READ_INTERVAL = 2.0          # Increase from 1.0

NOISE_MONITOR_READ_INTERVAL = 2.0   # Increase from 1.0

============================================================================

PART 17: GETTING HELP

============================================================================

View System Status:

-------------------

sudo systemctl status distress.service

View Detailed Logs:

-------------------

sudo journalctl -u distress.service --since today

Common Log Keywords:

--------------------

- "ERROR" - Error messages

- "WARNING" - Warning messages

- "BLE" - Bluetooth-related messages

- "ESP32" - Stress ball communication

- "Camera" - AI behavior detection

- "Noise" - Environmental noise monitoring

- "Temperature" - DHT22 sensor readings

- "Distress" - Distress detection alerts

Report Issues:

--------------

If you encounter problems, collect these details:

1. System logs (sudo journalctl -u distress.service -n 200)

2. Error messages

3. Hardware setup

4. Software versions (python3 --version, uname -a)

============================================================================

PART 18: REFERENCE - IMPORTANT FILE LOCATIONS

============================================================================

Python Virtual Environment:

----------------------------

your directory/tftenv/                         # Virtual environment

System Service:

---------------

/etc/systemd/system/distress.service        # Systemd service file

Hardware Configuration:

-----------------------

/boot/firmware/config.txt                   # Boot configuration (I2S, I2C)

Logs:

-----

sudo journalctl -u distress.service         # Service logs

============================================================================

PART 19: REFERENCE - IMPORTANT COMMANDS SUMMARY

============================================================================

Service Control:

----------------

sudo systemctl start distress.service       # Start

sudo systemctl stop distress.service        # Stop

sudo systemctl restart distress.service     # Restart

sudo systemctl status distress.service      # Status

sudo systemctl enable distress.service      # Enable auto-start

sudo systemctl disable distress.service     # Disable auto-start

View Logs:

----------

sudo journalctl -u distress.service -f              # Real-time

sudo journalctl -u distress.service -n 100          # Last 100 lines

sudo journalctl -u distress.service --since today   # Today's logs

sudo journalctl -u distress.service -b              # Since boot

System Control:

---------------

sudo reboot                                 # Reboot

sudo shutdown -h now                        # Shutdown

sudo shutdown -r now                        # Restart

Hardware Check:

---------------

arecord -l                                  # List audio devices

ls /dev/video*                              # List cameras

sudo i2cdetect -y 1                         # Scan I2C devices

gpio readall                                # Show GPIO status

iwconfig                                    # WiFi status

bluetoothctl                                # Bluetooth control

============================================================================

SYSTEM READY!

============================================================================

Your system should now be running. You can:

1. Connect via mobile app (scan for "CalmOrbHub")

2. Enable "Child Profile" to start monitoring

3. Give the stress ball to the child

4. Monitor real-time data in the app

5. Receive distress alerts automatically

For ongoing monitoring:

sudo journalctl -u distress.service -f

Questions? Check the README files in the GitHub

============================================================================

Last Updated: 15 January 2026

Version: 2.0.0 CalmOrb

============================================================================